# Darrieus VAWT Design & Construction (1–10 kW)

Executive Summary: This report reviews the principles and practical steps for designing, building, and operating a small (1–10 kW) Darrieus vertical-axis wind turbine (VAWT). We cover aerodynamic theory (lift vs. drag, tip-speed ratio, power coefficient, dynamic stall, cyclic loads), common rotor geometries (curved “egg-beater,” straight H-/Giromill, helical), airfoil choices and blade sizing, structural design (materials, fatigue, struts, bearings, tower), starting methods (Savonius hybrid, motor assist, passive twist), drivetrain and generators (direct-drive PM vs. geared induction, power electronics), mechanical subsystems (shaft, torque smoothing, no yaw needed), manufacturing methods, instrumentation/testing, safety/maintenance, modeling/simulation (CFD, blade-element), a build plan with BOM and cost estimates, and site/grid considerations. Key findings include: straight vs. helical blades trade self-start vs. peak Cp[1]; thick symmetric foils (NACA00xx, S8xx series) yield Cp≈0.3–0.4[2] but stall, while cambered/high-lift foils improve low-speed torque[2]; peak Darrieus power occurs at TSR≈4–6 (versus TSR≈1 for drag-based Savonius)[3][4]; Savonius–Darrieus hybrids cover a wider wind range[3]; direct-drive PM generators give high efficiency and reliability but need large rotors[5][6]; design options are summarized in tables below. Primary sources and recent studies are cited throughout.

## Aerodynamics: Lift vs Drag, TSR, Cp, Stall, Loads

Darrieus rotors harness lift from aerofoil blades moving through the wind. Unlike drag-based Savonius turbines (slow, self-starting, peak Cp at TSR≈1), lift-based Darrieus turbines run faster (TSR≈4–6) and achieve much higher Cp values. Reported peak power coefficients are typically in the 0.30–0.40 range for straight-blade designs[2]. For example, a straight H-rotor with symmetric S1046 profiles reached Cp≈0.35[2], while optimized cambered NREL foils (S823 series) gave a few-percent gain[2]. (Advanced modifications like slotted trailing edges or deflective flaps can boost low-TSR torque by 20–30%[7], and auxiliary ducts or flanges can push Cp above 0.6 in special cases.) In general, as wind speed increases, rotor power grows cubically (P≈½ρA V³Cp). Darrieus machines typically peak at higher TSR than Savonius: one study found a Savonius peak at TSR≈1 and a pure Darrieus at TSR≈6[3][4]. Hybrid designs (Savonius–Darrieus) attained maximum Cp≈0.26 at TSR≈2.7, benefiting from Savonius torque at low speeds and Darrieus efficiency at high speeds[3][8].

However, Darrieus blades undergo large cyclic angle-of-attack swings as they spin, causing dynamic stall and torque reversals at certain azimuths. These produce sharp negative-torque peaks and high vibration. Smoothing techniques (more blades, twisted/helical blades, pitch offset) can greatly reduce this cyclic loading. For example, adding blade twist suppresses dynamic stall and “flattens” the torque curve[9], while optimized airfoil shapes and pitch can eliminate negative torque intervals[10]. Reducing torque ripple is critical for structural life; experiments show that smoothing the torque can “significantly decrease torque ripple” and thus “lower cyclic loading reduces fatigue accumulation on the shaft, struts and blade attachments”[10]. These aerodynamic insights are applied in design (see below).

## Rotor Geometries

Darrieus VAWTs come in several geometries: curved (D-arrieus), straight-bladed (H-rotor or Giromill), and helical (twisted blades)[11]. Each has trade-offs:

- Curved (“eggbeater”) blades: One-piece curved blades attached to endplates and struts. High solidity gives good starting torque if not too heavy. Historically common (invented by Darrieus) because the curvature stresses blades in tension at speed. But they concentrate mass far from the hub, requiring robust support and heavier structure[12]. Curved blades can produce high Cp but incur strong bending loads and fabrication complexity.

- Straight H-rotor (Giromill): Vertical straight blades mounted on a central shaft via horizontal struts. Easier to manufacture (blades and struts are simple shapes) and less blade bending, but they suffer higher instantaneous torque ripple. They also typically need an external startup aid or motor because pure straight Darrieus often can stall at standstill (no inherent self-start)[12]. Most small Darrieus research uses 2–4 straight blades (with two or more struts)[12].

- Helical (twisted) blades: Blades twisted in a helical shape (e.g. 120° twist for three blades) form a “Savonius-shaped” swept area. Helical rotors blend Darrieus lift with an element of constant torque: they are known to self-start more easily than straight Darrieus[1]. Twist distributes the angle-of-attack variation along the blade, so dynamic stall is de-synchronized and peak-torque pulsations drop[9]. The trade-off is slightly lower peak Cp compared to the same blades un-twisted. Designers often choose helical blades for smoother operation, despite higher manufacturing effort.

Fig: Classic curved-blade Darrieus VAWT (two-bladed, egg-beater style). Blades are reinforced with end-plates and struts. (Image source: Wikimedia Commons)

## Blade Airfoil and Sizing

Airfoil selection is crucial. Symmetric aerofoils (e.g. NACA 00xx, SXXXX series) are common because they produce lift in both directions of rotation. Studies find e.g. NACA 0018 or NREL S823 yield solid performance: one CFD/wind-tunnel comparison showed symmetric S1046 giving Cp≈0.35 (straight 3-blade)[2]. Cambered (non-symmetric) foils (e.g. S815, EN0005) give higher starting torque (higher lift at low TSR) and can improve energy capture in turbulent air, but at the cost of added drag at high speed[2]. Many small designs use moderate-thickness (15–20% chord) profiles: thicker blades stall softer (trailing-edge stall) and are stiffer, whereas very thin foils (9–12%) can produce higher peak lift but suffer deep stall (sharp torque drop)[13].

Blade chord length and solidity (N⋅c/(πD)) are chosen by balancing weight, cost, and performance. A common guideline is solidity≈0.15–0.30 for three blades. For example, with three blades (N=3) and rotor diameter D=2R=4 m, a solidity σ=0.2 requires chord c ≈ σπD/N ≈0.84 m (solidity = 3⋅0.84/(π⋅4)≈0.20). In practice, high-solidity designs torque up at low speeds but stall out sooner. Blade span (height) often matches diameter or is larger to capture more wind. Detailed blade structural design (spar, shear webs, material layup) follows standard wind-turbine practice.

## Structural Design: Materials, Supports, Bearings

Blades: Wood (laminated Douglas-fir) was once common for small rotors, but modern practice favors composites. Fiberglass-reinforced polymer (GFRP) laminates are the norm for performance and fatigue life[14]. Sandia/NREL reports emphasize: “special emphasis is placed on fiberglass because this material is [currently] the material of choice for wind turbine blades”[14]. Wood can work for prototypes (laminated plywood/epoxy) but must be sealed against moisture and is heavier. Aluminum extrusions or sheets (with welded stiffeners) are an option for straight blades on budget turbines[15], but are heavier (reducing Cp). Carbon-fiber may be used for high-performance parts (very stiff but costly). In short, FRP composites (fiberglass or carbon) offer the best combination of strength, weight, and fatigue resistance[16].

Struts/Hub: Blades mount to the shaft via struts or endplates. Commonly, machined hubs or welded frames attach steel tubes (struts) to blade roots. Some integrate a short spar with the strut. Joints must carry bending loads and cyclic fatigue. For Darrieus, two sets of struts (at top and bottom of rotor) connecting to a central column/tower are standard[12].

Bearings: Vertical-axis turbines exert significant axial load (rotor weight and thrust on tower). Bearings must support both axial and radial loads at low rotational speed (few hundred rpm or less). Most designs use spherical roller or tapered roller bearings at the tower top (or both ends of the shaft)[17]. Frequent starts/stops (from gusts or shutdowns) demand good lubrication. Indeed, reliability data show “most wind turbine failures are bearing failures” due to lubrication issues[17]. Some experimental small VAWTs use a combination of one roller bearing and one plain (journal) bearing. Novel approaches like magnetic (near-frictionless) bearings have been tested: one design reported self-start at 2 m/s by using a magnetic bearing in lieu of one mechanical bearing[18]. In practice for DIY 1–10 kW, quality sealed roller bearings are simplest, but designers should consider axial preload and corrosion protection.

Tower/Support: Unlike HAWTs, VAWTs are often shorter and stiffer, but still need a tower or pedestal. Even a small turbine benefits from height (to clear ground turbulence). A general rule is hub height ~3–5× rotor diameter; local constraints apply. The tower also must handle bending from wind on the rotor. Guy wires are sometimes used for small towers. Foundations depend on site: a concrete pad or earth anchors. For cost estimating, NREL’s Giromill study (1980) gave tower material cost ≈$1–4/kg and foundation ~$300/m³ of concrete[19] (in 1980 dollars).

Fatigue: All turbine parts see cyclic stress. Blades experience high-cycle fatigue from turbulence and start/stop cycles. Designs should use fatigue-rated materials and joints. Minimizing stress range is key (smooth torque, avoid stall). FEA analysis is recommended to check blade deflection and strut fatigue. As noted, smoother torque curves dramatically reduce fatigue: “lower cyclic loading reduces fatigue accumulation on the shaft, struts, and blade attachments”[10]. Every design choice (solidity, blade thickness, support stiffness) impacts fatigue life.

## Starting Methods

Darrieus turbines do not self-start at low wind speeds since net torque can be negative at zero RPM. Common starting aids:

- Savonius hybrid: Add a small Savonius (drag) rotor concentric with or at one end of the Darrieus[3]. The Savonius generates high torque at zero speed and any azimuth, overcoming the deadband. Once spinning, the Darrieus lift-driven rotor takes over and the Savonius adds drag. Studies of Savonius–Darrieus hybrids (especially helical Savonius) show the rotor “combines both advantages”: the Savonius excites torque at low TSR while the Darrieus dominates at high TSR[3]. For instance, one “Helical Savonius–Helical Darrieus” design achieved positive torque at all azimuths and a moderate Cp peak (0.26) at TSR~2.7[8]. The drawback is extra cost/drag of the Savonius component. Table below compares starting options.

- Electric motor or clutch: A small motor (or temporary mechanical freewheel) can spin the shaft up past stall. This is simple and reliable for an experimental prototype; the motor is disengaged once the wind picks up. The downside is added complexity and energy use.

- Passive design changes: Helical blade geometry (as above) can significantly reduce start-up issues; helical rotors are “well-known for the least difficulty of self-starting”[1]. Some designs use overlapping (J-shaped) blades or slight preset pitch to improve torque in the downwind half-cycle. These measures help but may not guarantee self-start in very low wind.

Table: Starting Methods

| Method | Pros | Cons |
| --- | --- | --- |
| Savonius hybrid | Self-start at any wind; extends effective range[3] | Extra drag/weight at high speed; mechanical complexity |
| Electric motor | Reliable start regardless of wind | Requires power source; adds cost and maintenance |
| Blade twist (helical) | Improved low-speed torque; very smooth operation[1] | More complex blade fabrication; slight efficiency loss |
| Pitch control | (N/A for fixed-pitch Darrieus) | Darrieus are usually fixed-pitch (pitch adds complexity) |

## Drivetrain & Generator Options

The mechanical-to-electrical conversion involves either a gearbox or direct-drive, and a choice of generator type:

- Gearbox + generator: A gearbox (e.g. spur gears) can step up the slow rotor speed (~100–300 rpm) to a higher rpm suitable for a standard generator (1500–3000 rpm). This allows using a small, off-the-shelf induction or wound-rotor generator. Gearboxes are cheap and compact, but they introduce friction losses and reliability issues. In fact, industrial data shows gearboxes are a frequent failure point in wind turbines. If using a gearbox, ensure it is rated for the torque and has good lubrication. The gearbox should also be chosen or designed to handle the VAWT’s torque pulsations.

- Direct-drive (no gearbox): The rotor shaft connects straight to the generator. This requires a generator designed for low speed/high torque (many poles or large diameter). For example, an axial-flux or pancake PM generator can be built with dozens of poles to generate power at <200 rpm. The advantage is eliminating gearbox losses and failures. The downside is size and cost: direct-drive generators tend to be large-diameter and heavy. As one report notes, “the direct-drive concept is more superior in terms of energy efficiency, reliability, and design simplicity”[5], but it requires very rigid air-gap structure[20][21]. Direct-drive PM machines (with permanent magnets) have high power/weight and efficiency (no slip rings or excitation supply)[22]. ORNL even promotes modular direct-drive PM stators for small wind turbines to maximize efficiency and reliability[6].

- Generator type: For small wind, Permanent-Magnet Synchronous Generators (PMSG) are popular because they avoid external excitation and can be made multi-polar for direct drive[23]. Induction generators (either squirrel-cage or doubly-fed) can be used: a cage machine is simple and robust but requires capacitors for self-excitation or an inverter to supply reactive power. DFIG systems (with slip rings) allow variable-speed control but are complex. In practice, many stand-alone small turbines use PMSG for its simplicity and then use an inverter to interface to load/grid[23]. With an induction generator and no inverter (stand-alone), you must supply reactive excitation (capacitor bank) and the turbine output is fixed-frequency (best if geared to a standard generator rpm).

- Power electronics: Grid-tied or battery-based systems need power electronics. A typical chain is: generator output → AC/DC rectifier → DC/DC converter (MPPT stage) → inverter (to grid or battery). Controllers are used to maximize power (MPPT), regulate DC link, and meet grid standards. For off-grid/battery systems, a charge controller is used. If grid-tying, anti-islanding protections and compliance with IEEE 1547/UL1741 (in the US) are needed. As Abarzadeh et al. note, a small wind system often consists of a PMSG, AC/DC rectifier, DC/DC MPPT, and inverter[24][23].

Table: Generator/Drivetrain Options

| Option | Pros | Cons |
| --- | --- | --- |
| Gearbox + Induction | Uses common, cheap generator; compact | Gearbox adds maintenance; reactive power needed (cap-tank) |
| Gearbox + PMSG/SG | Allows standard generator; moderate efficiency | Gearbox issues; slip-ring needed for SG (brushes), PMSG needs capacitor/inverter for grid |
| Direct-drive PMSG | No gearbox losses; high efficiency; simple shaft alignment[6] | Large/expensive generator; heavy bearings; rigid construction needed[20] |
| Direct-drive SG (excited) | No permanent magnets; on/off control of excitation | Requires slip rings; lower P/W ratio; brushes maintenance |
| DFIG (with gearbox) | Partial inverter; can regulate reactive power | More complex; brushes/slip rings; not common in small VAWT |

## Mechanical Subsystems

- Shaft: The rotor shaft carries combined aerodynamic torque. A multi-blade rotor can induce bending moments if asymmetrical. The shaft must be sized for torsion plus bending (from wind thrust) with ample factor of safety. A two-bearing support (top and bottom) is recommended; alignment is critical.

- Torque smoothing: As above, three or more blades and helical twist can smooth the output. A balanced 3-blade H-rotor tends to have much less vibration than a 2-blade. Some designs add a flywheel or inertia for further smoothing.

- Yaw: Vertical-axis machines inherently face all wind directions, so no yaw mechanism is required. The turbine is omni-directional.

## Manufacturing and Fabrication

Blades: For small runs, blades can be made by hand lay-up of fiberglass in molds (male/female wooden forms) or by using extruded aluminium channels bonded/welded into aerofoil shapes. Wooden glued-laminate blades (like boat hull construction) are another DIY option. Helical blades require jigs or segmented assembly. If using extruded sections, welding fixtures ensure accuracy.

Spars and Struts: Simple steel tube struts can be welded to hubs using welding jigs. Use high-strength steel (e.g. 4130 chromoly or structural steel) for struts if space allows; tube lighter for same stiffness. All welded joints should be smooth, without notches, to avoid crack initiation.

Shaft and Bearings: The shaft can be steel round bar; machine flats or splines for blade attachment if needed. Bearings should be preloaded to prevent slop. Drivetrain housings (if any) should allow for lubrication and maybe a bearing cover.

Generator Housing: If directly coupling, mount the stator stationary (e.g. on tower or base) and rotor attached to shaft. For geared, align pinion/crown with minimal backlash and shim to maintain gear clearance.

Tools: Standard metalworking tools suffice: saws, welder, drill press, lathe (optional for hubs). If composite: fiberglass cloth, epoxy resin, vacuum pump (for VARTM) or clamps. CNC cutting can cut accurate aerofoil blanks. Safety tooling (saws, lifts) for blade handling is important.

## Instrumentation & Testing

- Prototype testing: Use an anemometer (calibrated) to measure wind speed at hub height. Use a digital tachometer for RPM. Torque can be measured indirectly via power (electrical output) or using a torque transducer or calibrated spring. Instrumentation typically includes voltage/current sensors on the generator, and possibly strain gauges on blades or shaft for stress measurement.

- Wind tunnel: For sub-scale models, wind tunnel tests yield Cp vs TSR curves. Many VAWT studies use low-speed tunnels with dynamometer loads. For a DIY, testing in the open is likely simpler: measure power output at various steady winds (e.g. using variable electrical load or dump resistor to vary torque).

- Data collection: Important data: wind speed vs time, rotor speed vs time, mechanical torque (or electrical power), vibration (accelerometers), temperatures (bearings/generator). Collect enough data to chart the power curve and identify any resonances.

- Safety during testing: Start with brief low-speed tests. Always have a quick-stop brake (friction or mechanical) in case of overspeed or failure. Keep all personnel away from the plane of rotation. If testing indoors, ensure adequate distance.

## Safety, Maintenance, and Failure Modes

Safety: Include a brake or furling mechanism for overspeed protection. A simple mechanical band brake on the rotor shaft or vertical stall-redirector vanes (passive) can limit RPM. Disconnects must be installed for the electrical side. As a rotating machine, the turbine must be guarded on all sides to prevent contact. Lightning protection (grounding the structure) is advised for tall installations.

Maintenance: Regular lubrication of bearings/gearbox (if any). Inspect blades and welds yearly for cracks. Check electrical connections and inverter logs (if used). Clean insects and debris from blades.

Failure Modes: Blades or joints can fail in extreme gusts if overstressed. Fatigue cracks can grow in welds or composite laminates. Gearboxes (if used) can strip teeth under shock. Generators can overheat if stalled. Design safety factors (>1.5–2) and quality components mitigate these.

## Performance Modeling & Simulation

Several methods predict Darrieus performance:

- CFD/FEA: High-fidelity Computational Fluid Dynamics (e.g. 2D/3D RANS) can simulate flow around VAWT blades and wake interactions. Published CFD studies (using k-ε turbulence and multiple reference frames) have successfully predicted Cp values (e.g. ≈0.32 at TSR 3.5 for a 3-blade H-Darrieus[25]). CFD can capture dynamic stall and 3D effects but requires expertise.

- Analytical/Empirical: Simplified blade-element models (BEM modified for VAWT) or vortex models exist. For example, the “double-multiple streamtube” method treats each azimuth separately. Empirical coefficients or lookup from experiments can be used. Some studies calibrate models using wind tunnel data. No single formula perfectly captures VAWT performance, so combining CFD and test data is best.

- CFD results: As a data point, Hao et al. reported in a systematic study that a 3D CFD model of a 3-blade Darrieus achieved a Cp of ~0.32–0.36 at optimum TSR (~3.5)[25]. This aligns with wind-tunnel trends. Designers should check models against at least one measured point if possible.

- Dynamic simulation: For control/structural analysis, couple aero models to drivetrain dynamics to study start-up and shutdown, torsional modes, etc.

## Build Plan (Steps & Timeline)

flowchart TD
    A[Gather requirements & site wind data] --> B[Preliminary design & sizing]
    B --> C[Airfoil selection & CAD modeling]
    C --> D[Material sourcing & procurement]
    D --> E[Fabricate rotor blades & struts]
    E --> F[Machine hub, shaft, and support structure]
    F --> G[Assemble rotor and drivetrain components]
    G --> H[Install generator & electronics]
    H --> I[Ground installation of tower/support]
    I --> J[Test in field or wind tunnel]
    J --> K[Tune controller & commissioning]

Typical timeline (DIY context) might be 6–12 months from design start to commissioning, depending on shop resources. Key milestones: blade completion, shaft assembly, generator integration, preliminary testing, grid safety check.

## Example Calculations

- Blade chord (solidity): For N=3, rotor radius R, desired solidity σ, chord ≈ σ·π·2R /3. E.g. R=2 m, σ=0.2 ⇒ c ≈0.84 m.

- Rotor area: Swept area A≈2R·H (cylindrical projection). If R=2 m, H=4 m, A≈16 m².

- Power vs Wind Speed: Using P=½ρAV³Cp. For ρ=1.225 kg/m³, A=16 m², Cp=0.30:

- At V=6 m/s: P≈0.5×1.225×16×6³×0.30≈730 W.

- At V=10 m/s: P≈3,000 W. Thus a 4 m×4 m rotor could hit ~3 kW at 10 m/s with Cp≈0.3. (Graphs of P vs V are typically cubic.) A real prototype power curve will start at cut-in (~3–4 m/s) and level off or curtail by ~12–15 m/s.

## Cost, BOM, and Scaling

Based on historical studies, small wind turbine costs scale with mass and complexity. NREL’s Giromill analysis (1980) provides rough material cost ranges: rotor (blades+hub) $5–$37/kg, drive $3–$11/kg, electrical/generator $5–$22/kg, controls $24–$79/kg[26]; tower $1–$4/kg; foundation about $300/m³ concrete[19]. These give ballpark: a ~100 kg turbine might cost a few thousand dollars of parts.

A sample Bill of Materials (BOM) for a 3 kW prototype might be:

| Component | Qty | Est. Unit Cost | Total Cost |
| --- | --- | --- | --- |
| Blades (fiberglass) | 3 | $100 each | $300 |
| Hubs & struts | 1 | $200 | $200 |
| Shaft & bearings | 1 | $250 | $250 |
| Tower & foundation | 1 | $500 | $500 |
| Generator (PM, 5 kW) | 1 | $1,200 | $1,200 |
| Inverter/Controller | 1 | $400 | $400 |
| Wiring & hardware | - | $150 | $150 |
| Total |  |  | $3,000 |

(Costs are approximate; labor and machining may double total. Larger turbines have somewhat higher costs but benefit from economies of scale. Cost of energy typically scales roughly inversely with rotor area and mass.)

## Site Selection & Grid Tie

Choose a site with high average wind (site class 3+ or ~6 m/s at hub). Avoid nearby obstructions (trees, buildings) within ~3× turbine height (see DOE small wind siting guides). For hilly terrain, position the turbine on or windward of a ridge for wind acceleration[27]. A professional site assessment (anemometry over ≥1 year) is ideal but not strictly necessary for a DIY unit.

For grid connection, check local utility interconnection rules. In the U.S., small turbines (<20 MW) typically follow IEEE 1547 and UL1741 standards. You may need a certified inverter. The interconnection process often requires submitting a single-line diagram and proof of anti-islanding protection. Some regions allow simplified “fast-track” process for <10 kW. If off-grid, a battery or dump load will be needed. Regulatory loads for small wind are generally light (often a building permit and electrical inspection). Always include proper disconnects and fuses on the DC/AC side.

## References

This report references recent research and authoritative sources. Key citations include empirical and computational studies of Darrieus aerodynamics[2][10][9], rotor design comparisons[3][8], materials/fatigue data[16][14], drivetrain and generator analyses[5][23], and DOE/NREL guidance on small wind systems[26][19]. These underpin the design trade-offs and recommendations summarized above.

[1] [9] Power performance and self-starting features of H-rotor and helical vertical axis wind turbines with different airfoils in turbulence - ScienceDirect

https://www.sciencedirect.com/science/article/abs/pii/S0196890423007513

[2] [7] [13] [25] A comprehensive review on H-type Darrieus wind turbine: aerodynamics, blade profile, CFD simulations | Journal of Engineering and Applied Science | Springer Nature Link

https://link.springer.com/article/10.1186/s44147-026-00968-x

[3] [4] [8] Hybridization of a Micro-Scale Savonius Rotor Using a Helical Darrieus Rotor

https://www.mdpi.com/2311-5521/10/3/63

[5] [20] [21] [22]  Direct Drive Wind Turbine vs. Geared Drive Wind Turbine

https://www.windustry.com/direct-drive-wind-turbine-vs-geared-drive-wind-turbine.htm

[6] Direct Drive Modular Permanent Magnet Arc Generator | ORNL

https://www.ornl.gov/technology/202305420

[10] Range-Wide Aerodynamic Optimization of Darrieus Vertical Axis Wind Turbines Using CFD and Surrogate Models | MDPI

https://www.mdpi.com/2674-032X/6/1/2

[11] Different types of Darrieus rotor VAWT   | Download Scientific Diagram

https://www.researchgate.net/figure/Different-types-of-Darrieus-rotor-VAWT_fig1_313893605

[12] coeng.uaa.alaska.edu

https://coeng.uaa.alaska.edu/innovation/wp-content/uploads/2020/08/2015-Vertical-Axis-Wind-Turbine-Strut-and-Blade-Design-for-Rural-Alaska-Final-Report-VAWT.pdf

[14] emsb063.tmp

https://digital.library.unt.edu/ark:/67531/metadc621567/m2/1/high_res_d/12694.pdf

[15] [16] Blade material influence on the performance of H-Darrieus vertical axis wind turbines: a review | Request PDF

https://www.researchgate.net/publication/400669862_Blade_material_influence_on_the_performance_of_H-Darrieus_vertical_axis_wind_turbines_a_review

[17] [18] Functionality of Bearings in the Shafts of a Vertical-Axis Wind Turbine

https://www.mdpi.com/1996-1073/17/20/5189

[19] [26] Giromill Overview

https://docs.nrel.gov/docs/legosti/old/263.pdf

[23] [24] Power Electronics in Small Scale Wind Turbine Systems | IntechOpen

https://www.intechopen.com/chapters/39956

[27] Small Wind Guidebook | Department of Energy

https://www.energy.gov/cmei/wind/windexchange/windexchange/small-wind-guidebook